# 3x3 CNN Convolution Accelerator for Edge-AI Vision Application

## IEEE SSCS Egypt Chapter - 2026 Student Design Competition

## Team: Amir Diaa EL-Din Abouelela Hamza

## University: Cairo University, Faculty of Engineering, Computer and Communications Department

## Abstract

## 1.Accelerator Architecture

The accelerator accepts 32x32 gray-scale images (1024 pixels) and outputs a feature map of 1024 pixels after applying a 3x3 same (zero padded) convolution. The throughput is one output pixel per clock cycle. The input precision is 8-bit unsigned integer Q8.0 and the coefficients are 8-bit signed integers. Final output is a 16-bit signed integer which is saturated. ReLu is a build time parameter and is applied after saturation. By default, it is off.

The kernal is programmable through coefficients loaded through write_en / write_addr[3:0] / data_write[7:0], write_addr = 3*row + col (0..8), row-major, same order as the taps, addresses 9..15 ignored. Writes are ignored while busy flag is high. However, it is reloadable between frames (proven by the back-to-back run, sobel_x then sharpen, kernel swapped while frame 1 drains).

The architecture type is a streaming line-buffer sliding-window convolution, fully parallel 9-multiplier datapath, 3 pipeline register stages after the window and1.0 output pixel per cycle.

There is strict control/datapath split. control_fsm (rtl/control_unit.sv) owns every counter and flag: state, in_cnt (10b), out_r/out_c (5b each), drain_cnt (6b), four registered edge flags. The window generator, line buffers, coeffiecient  registers , 8x8-bit multiplier and the adder tree hold no position state. The window generator has no counters and no valid output generation mechanism, all state is decided by the control unit.

The Control-to-datapath signals are only as follows: en and the four edge flags into window_gen, busy into coeff_reg (write lock), win_valid into the 3-stage valid chain.

The Datapath in one line: 9 taps (9x8b) x 9 coefficients (9x8b) -> 9 LUT multipliers -> stage-1 register prod_r 9x16b -> three 3-input row adders -> stage-2 register row_r 3x18b -> one 3-input 20-bit adder -> saturate 20b->16b -> ReLU mux -> stage-3 register pixel_out.

The Valid chain runs level with the data: win_valid -> v_prod -> v_row -> valid_out. The pipeline registers are free-running, so a valid_in stall becomes a bubble in valid_out exactly 3 cycles later, never corruption.

The window generation is done by two 32-deep 8-bit line buffers in series (LB0 from input_in, LB1 from LB0) + 9 tap registers a..i + combinational zero-pad masks driven by the edge flags (section 6).

Each video frame processes through a five-state FSM: IDLE, FILL, STREAM, DRAIN, and DONE.

The sequence begins in FILL, consuming 34 input-only cycles to fill the pipeline before transitioning to STREAM, which concurrently ingests inputs and produces outputs until the 1,024th pixel is consumed. The FSM then enters DRAIN for 34 output-only cycles to flush remaining data while ignoring new inputs, followed by a single-cycle DONE state that asserts frame_done before returning to IDLE. The enable signal mirrors input validity (en = valid_in), except during DRAIN where it is forced high (en = 1) and DONE where it is deactivated (en = 0). Because the fill and drain durations are balanced at 34 cycles each, total frame inputs and outputs equal exactly 1,024 pixels.

The top-level core interface comprises 42 pins allocated across four signal groups:

- System & Status (4): clk, rst_n (active-low, synchronous), busy

- Data Input (9): input_in[7:0], valid_in

- Write Port (13): write_en, write_addr[3:0], data_write[7:0]

- Data Output (17): pixel_out[15:0] (signed), valid_out

The module operates without a ready backpressure signal. When valid_in drops on any cycle, the clock enable (en) gates line buffers, tap registers, and internal counters simultaneously, freezing the current window coherently.

Frame delivery is managed by the source logic:

- The source streams exactly 1,024 pixels per frame and stalls subsequent frames until busy drops

- The busy flag asserts during active processing (busy = !IDLE && !DONE).

- To optimize throughput, busy deasserts 3 cycles before the final valid_out pulse, as the final pixel window is already committed to the processing pipeline.

Latency & Throughput

- Latency: First valid_out occurs 37 cycles after the first accepted pixel (34 window-fill cycles + 3 internal pipeline stages).

- Peak Throughput: 1.0 output pixel/cycle, gapless within an active streaming frame.

- Sustained Throughput: 0.965 pixels/cycle per frame (1,024 outputs over 1,061 total cycles, accounting for fill and drain overhead).

Figure of Merit (FoM) Architectural Levers

- Lever 1 — Peak Throughput (1.0 px/cycle): Nine parallel multipliers process all nine window pixels simultaneously every cycle. A time-multiplexed 3-multiplier design (1 output every 3 cycles) was rejected; pre-implementation estimates indicated this would trade a 3× throughput loss for only a ∼1.5× area reduction. This choice also secures the throughput performance bonus.

- Lever 2 — Zero BRAM Usage (100x Penalty Avoidance): Line buffers are implemented in fabric shift registers, inferring 16 SRLC32E primitives (16 LUTs total). Mapping to even a single Block RAM tile (100 LUT-equivalents penalty) would increase the area denominator from 879 to 963 LUTs (+9.6%).

- Lever 3 — Zero DSP Blocks (50x Penalty Avoidance): Enforced via the use_dsp = "no" attribute on mult8x8 (rtl/multiplier.sv) and -max_dsp 0 in fpga/build.tcl. The nine 8u×8s multipliers consume 496 LUTs combined (52–59 LUTs each). Standard DSP48E1 instantiation would cost 450 LUT-equivalents (9×50), making the trade-off roughly area-neutral vs. 496 LUTs. The soft-logic approach was retained to eliminate fractional utilization of 25×18 DSP slices, maintain a uniform logic fabric/power model, and preserve vendor-neutral RTL via a single attribute.

- Full Programmability Preserved: Constant-coefficient multipliers were rejected. Although hardcoding the standard kernel values (0,±1,±2,±4,5) could save ∼300 LUTs, runtime dynamic programmability (Requirement Item 3) would be forfeited.

Resource Breakdown (879 Routed LUTs)

- Multipliers (9x): 496 LUTs

- Window Generator: 132 LUTs (116 Logic + 16 SRL)

- Coefficient Registers: 120 LUTs

- Top-Level Logic: 90 LUTs (Adder Tree, Saturation, ReLU)

- Control FSM: 80 LUTs

Headline Results Overview

(Forward-referenced to Sections 12–14)

- Target Device & Toolchain: AMD/Xilinx Zynq-7000 xc7z020clg400-1 (PYNQ-Z2), Vivado 2025.2

- Target Clock: 125 MHz (8.000 ns period constraint)

- Resource Utilization: 879 LUTs, 441 FFs, 0 DSP, 0 BRAM, 42 IOB, 1 BUFG

- Timing & Frequency: WNS +0.742 ns, WHS +0.153 ns (0 failing endpoints / 602); Fmax=137.8 MHz

- Power (SAIF-Annotated, Medium Confidence): 0.158 W total (0.107 W static, 0.052 W dynamic)

- Overall FoM (@ 125 MHz): FoM=1.00.158 W×879 LUTs=7.20×10-3

Verification Summary

(Forward-referenced to Sections 9–11)

Reference Model: golden_model/golden.py (NumPy-based reference implementation).

Regression Suite: 22 regression runs completed with 100% pass rate (7 kernels tested across clean, fixed-stall, and random-stall conditions, plus a back-to-back frame run). Both RELU=0 and RELU=1 DUT configurations were evaluated against identical stimulus.

Hardware Verification: All 14 generated hw_out/ files are byte-identical to the corresponding golden_model/hex/ target outputs.

Module & File Structure Map

rtl/cnn_pkg.sv: Global parameters and package definitions.

rtl/control_unit.sv: Control logic (control_fsm).

rtl/window_gen.sv: 3×3 sliding window generator.

rtl/line_buffer.sv: Line buffering shift-register logic.

rtl/coeff_reg.sv: Programmable kernel coefficient storage.

rtl/multiplier.sv: Instantiates mult8x8 signed/unsigned multiplier units.

rtl/top.sv: Top-level integration (9 multiplier instances, adder tree, saturation logic, ReLU, 3-stage pipeline registers, and valid-signal propagation chain).

Architectural Bonuses Claimed

- Pipelined High Throughput: Sustains one output pixel per cycle during active streaming.

- Multiple Kernel Support: Dynamic runtime programmability; verified across 5 realistic kernels reloaded between frames.

- Integrated ReLU Activation: Parameterized activation layer (RELU), verified across all 7 test kernels.

- Simulation Edge-Detection Demo: Full visual validation rendered directly from hw_out/ simulation outputs.

The bitstream fpga/top.bit was compiled for the target PYNQ-Z2 pinout to claim the FPGA board demo bonus.

Architectural Assumptions

- Fixed Resolution: Processing is strictly structured around a 32×32 fixed image size.

- Padding Mode: Employs zero-padded "same" convolution yielding 1,024 outputs per frame, rather than "valid" convolution (30×30).

- Coefficient Ordering: The coefficient update port expects data in strict row-major order (c00 through c22).

- FoM Power Units: Power metric within the Figure of Merit denominator is interpreted in Watts (W).

- Clock Frequency Target: Figure of Merit evaluation is normalized to a 125 MHz system operating clock.

- Handshaking Strategy: No backpressure ready signal is provided; flow control relies entirely on source-side frame gating based on the busy signal.

- Timing Domain Scope: External I/O timing delays are excluded from internal FPGA timing closure calculations.

- Write Port Arbitration: The coefficient write interface is ignored while the core asserts busy.

- Data Ingestion: Input stream follows standard raster order at 1 pixel/cycle peak, with support for arbitrary valid_in stall cycles.

- Kernel Set: Evaluated kernel configurations conform to standard image-processing convolution masks.

Figure 1.1: Valid out is high throughout the 1024 cycles , 1px/cycle

Table 1: Design Parameters:

| Parameter | Value | Kind | Note |
| --- | --- | --- | --- |
| IMG_W | 32 | Fixed | Image width (pixels) |
| IMG_H | 32 | Fixed | Image height (pixels) |
| KSIZE | 3 | Fixed | 3x3 kernel size |
| NTAP | 9 | Derived | Total taps (KSIZE^2) |
| N_MAC | 9 | Fixed | Fully parallel MAC units |
| LB_DEPTH | 32 | Derived | Equal to IMG_W (1 full row) |
| IN_W | 8 | Fixed | Unsigned Q8.0 input width |
| COEF_W | 8 | Fixed | Signed coefficient width |
| COEF_FRAC | 0 | Toggle | Fractional bits (7 = Q1.7 rebuild) |
| OUT_W | 16 | Fixed | Signed saturating output width |
| RELU_EN | 0 | Toggle | Top parameter (RELU) enable |
| PROD_W | 16 | Derived | Product width (IN_W + COEF_W) |
| ACC_W | 20 | Derived | Accumulator width |
| OUT_MAX | +32767 | Derived | Max signed 16-bit ceiling |
| OUT_MIN | -32768 | Derived | Min signed 16-bit floor |
| FILL_CYCLES | 34 | Derived | Equal to LB_DEPTH + 2 |
| NPIX | 1024 | Derived | Total frame pixels (IMG_W x IMG_H) |
| DRAIN_CYCLES | 34 | Derived | Equal to FILL_CYCLES |

2. Block Diagram

The block diagram represents the hardware CNN convolution pipeline. Window_gen generates the required 3×3 pixel windows, which are processed by the 9 multipliers and adder tree using the coefficients stored in coeff_reg. A Control FSM coordinates the data flow and timing between the blocks. The resulting convolution value is then passed to the Sat/ReLU block for saturation and ReLU activation, producing the final pixel_out along with the valid_out signal.

Figure 2: Block Diagram; Dashed lines are pipeline boundaries

## 5. Memory Organization

Memory Architecture & Storage Optimization

- Block RAM Usage: 0 of 140 tiles used (RAMB36=0, RAMB18=0). Distributed LUT RAM is also 0. The sole memory-class primitive across the entire design is 16 × SRLC32E (LUT as Shift Register), per fpga/reports/utilization.rpt (Sections 1 & 3).

- Streaming Image Buffer Strategy: The input image is never fully stored on-chip. Maximum active state at any time is exactly 2 rows + 9 pixels: two 32-deep line buffers (2×32×8=512 bits) and nine 8-bit tap registers (72 bits). Every output pixel requires only one external read; the 3×3 window pixels are provided entirely by internal registers with zero re-reading.

- Line Buffer Logic (rtl/line_buffer.sv): Implemented as an 8-bit wide×32-deep shift register with clock enable (en) gating and synchronous reset. Data enters at sr[0] and outputs at sr[DEPTH-1]. The structural delay is fixed at 32 cycles without needing write pointers, read pointers, or address comparison logic.

- SRL Inference Mechanics: Despite the synchronous reset, Vivado infers SRL primitives because only the final stage (sr[DEPTH-1]) is ever read. This maps the buffer chain directly into LUT shift registers (fpga/reports/RESULTS.md), saving 512 flip-flops for a total cost of just 16 LUTs.

- Synthesis & Cell Evidence: The synth.log "Static Shift Register Report" confirms two rows—u_wg/LB1/sr_reg[0][7] and u_wg/tap_reg_reg[2][7]—each with Length 30, Width 8, requiring 8 × SRLC32E primitives per line (16 SRLC32E total). This matches the "Report Cell Usage", utilization.rpt, and utilization_hier.rpt reports.

- SRLC32E Interpretation: An SRLC32E primitive turns a single LUT6 into a 32-bit shift register. Eight cells in parallel handle one 8-bit wide row. The "Pull out last Reg = YES" synthesis setting indicates Vivado placed the final reset-bearing stage into a dedicated flip-flop while shifting the preceding 30 stages into SRL logic.

- Primitive Alignment: A row depth of 32 matches the 32-bit capacity of the SRLC32E primitive precisely (1 LUT/bit). A 29-deep buffer would yield zero area savings, as it would still require 8 LUTs per line.

- The 100× BRAM Penalty Justification: Block RAM incurs a 100× LUT-equivalent penalty per tile under the FoM rules. Implementing both line buffers in a single BRAM tile (storing 256 bits, just 1.4% of an 18 kbit tile) would inflate the area denominator from 879 to 879-16+100=963 LUTs (+9.6% arithmetic penalty). Furthermore, BRAM would demand address counters, output registers, and read-latency alignment logic that the fixed shift register avoids entirely.

Register & Control State Breakdown

- Tap Registers: 9×8 bits=72 FFs inside window_gen. These are enable-gated and shift horizontally across rows (a←b←c, d←e←f, g←h←i). All 9 taps are read simultaneously by the multipliers every cycle, ruling out RAM structures.

- Coefficient Storage (rtl/coeff_reg.sv): 9×8 bits=72 FFs. Controlled via a single write interface (write_en, write_addr[3:0], data_write[7:0]) guarded by write_en && !busy && write_addr < 9. Provides 9 parallel output buses with no multiplexing. The routed cost of 120 LUTs reflects address decode fan-out across 9 byte-enables rather than storage logic.

- Datapath Pipeline Registers (rtl/top.sv): Free-running without enables. Consists of multiplier product registers prod_r (9×16=144 FFs), accumulator stage registers row_r (3×18=54 FFs), output register pixel_out (16 FFs), and valid bit propagation pipeline v_prod/v_row/valid_out (3 FFs), totaling 217 FFs.

- Control FSM State (rtl/control_unit.sv): Consists of in_cnt (10 FFs), out_r (5 FFs), out_c (5 FFs), drain_cnt (6 FFs), 4 edge condition flags, and state encoding (35 FFs routed after Vivado one-hot re-encoding).

- Flip-Flop Totals: 441 total routed FFs across the design, composed of 217 (top-level pipeline) + 72 (coeff_reg) + 35 (control_fsm) + 117 (window_gen, including 72 taps). Net datapath storage spans 512+72+72=656 bits with 0 BRAM and 0 Distributed RAM.

Control & Reset Strategy

- Dual Enable Scheme: Elements maintaining spatial window context (line buffers, tap registers, FSM counters) are gated by en to freeze the window coherently during stalls. Elements downstream of the window generator run free and are qualified by the traversing valid bit pipeline. The design synthesizes into 16 unique control sets.

- Reset Behavior: Every register across all modules initializes on synchronous active-low rst_n. Correct edge zero-padding is enforced independently by spatial masking logic rather than register reset states.

- Scalability Note: On-chip memory scales strictly with image width (2×IMG_W×8 bits) rather than image height. Expanding to wider image resolutions cascades additional SRLC32E primitives without forcing a transition to Block RAM.

### Table: On-Chip Storage Breakdown

| Element | RTL Location | Size (bits) | Routed Implementation | Enable | Why Not BRAM? |
| --- | --- | --- | --- | --- | --- |
| LB0 | window_gen / line_buffer | 256 (32×8) | 8× SRLC32E | en | 100× penalty vs. 8 LUTs; no address logic required |
| LB1 | window_gen / line_buffer | 256 (32×8) | 8× SRLC32E | en | Same as LB0 |
| Tap Registers (a…i) | window_gen (tap_reg) | 72 | FFs | en | Requires 9 parallel reads per cycle |
| Coefficient Registers | coeff_reg | 72 | FFs | write_en && !busy | Requires 9 parallel reads per cycle |
| prod_r | top | 144 | FFs | Free-running | Datapath pipeline register |
| row_r | top | 54 | FFs | Free-running | Datapath pipeline register |
| pixel_out + Valid Chain | top | 19 (16+3) | FFs | Free-running | Output register and valid bit propagation |
| Control Counters / Flags / State | control_fsm | 35 | FFs | Mixed | Control logic state, not a memory array |

Total: 441 FFs, 16 SRL LUTs, 0 BRAM, 0 LUTRAM (Sources: utilization.rpt, utilization_hier.rpt, synth.log)

### Table: Memory-Primitive Synthesis Evidence

| Report | Line / Section | Value |
| --- | --- | --- |
| synth.log | Static Shift Register Report | 2 chains, Length 30, Width 8, 8 SRLC32E each |
| synth.log | Report Cell Usage | SRLC32E: 16 |
| utilization.rpt | Section 1 (Slice Logic) | LUT as Shift Register: 16, LUT as Distributed RAM: 0 |
| utilization.rpt | Section 3 (Memory) | Block RAM Tile: 0 / 140 |
| utilization_hier.rpt | Module Hierarchy (u_wg) | SRLs: 16, RAMB36: 0, RAMB18: 0 |

## 6. Window Generation Methodology

## 6. Line-Buffer and Window-Generation Method

### 6.1 Structural Architecture and Tap Topology

The sliding 3×3 window generator (rtl/window_gen.sv) transforms the serial raster input stream into nine parallel spatial pixel taps. The unit contains no internal position counters and generates no valid signals; it operates strictly under the control of the global clock enable (en).

The datapath structure flows as follows:

- input_in feeds tap i directly and enters line buffer LB0.

- LB0 output (in_f) feeds tap f and enters line buffer LB1.

- LB1 output (in_c) feeds tap c.

- Each tap row forms a 3-register horizontal shift chain:

  - Top Row: c→b→a

  - Middle Row: f→e→d

  - Bottom Row: i→h→g

| Tap | Index | Window Position | Fed From | Delay behind input_in (cycles) | Pixel when Center = (r,c) |
| --- | --- | --- | --- | --- | --- |
| a | 0 | Top-Left | Tap b | 67 | r-1,c-1 |
| b | 1 | Top-Center | Tap c | 66 | r-1,c |
| c | 2 | Top-Right | LB1 out (in_c) | 65 | r-1,c+1 |
| d | 3 | Middle-Left | Tap e | 35 | r,c-1 |
| e | 4 | Center | Tap f | 34 | r,c |
| f | 5 | Middle-Right | LB0 out (in_f) | 33 | r,c+1 |
| g | 6 | Bottom-Left | Tap h | 3 | r+1,c-1 |
| h | 7 | Bottom-Center | Tap i | 2 | r+1,c |
| i | 8 | Bottom-Right | Top Input (in_in) | 1 | r+1,c+1 |

When center tap e holds target pixel r,c, the window contains spatial positions r-1…r+1,c-1…c+1. The full spatial window for position r,c becomes available at the taps the exact cycle pixel r+1,c+1 enters the chip. Stride-1 operation occurs naturally: each enabled cycle shifts the window one column right along the raster path.

6.2 Line-Buffer Depth Rationale (DEPTH=32)

Vertical Alignment Mechanics: The line buffers supply full 1-row (32-pixel) vertical delays. The 3-register tap chains branch in parallel off the line buffer outputs (input_in feeds LB0, in_f feeds LB1). They do not sit in series with the vertical line storage..

### 6.3 Pipeline Priming, Latency, and Edge Zero-Padding

The window generation phase requires 34 fill cycles (FILL_CYCLES=LB_DEPTH+2):

- Pixel 0,0 enters tap i on shift cycle 1.

- It exits LB0 after 32 additional shifts, entering tap f on shift cycle 33.

- It shifts into center tap e on shift cycle 34.

The center tap e holds pixel 0,0 at cycle 34, yielding the first valid spatial window. The pipeline does not wait 66 cycles for LB1 to fill; LB1 feeds the top row (a,b,c), which is masked to zero during output row 0 by border padding logic. Following cycle 34, one new spatial window is emitted every enabled cycle for all 1,024 frame pixels.

#### Combinational Zero-Padding Masking

"Same" convolution padding (32×32 output matrix) is implemented using combinational multiplexing masks immediately after the tap registers, preventing buffer-flush operations:

| Flag | FSM Condition | Taps Zeroed | Target Boundary | Windows Affected |
| --- | --- | --- | --- | --- |
| top_edge | nxt_r == 0 | a, b, c (0, 1, 2) | Row 0 | 32 |
| bottom_edge | nxt_r == 31 | g, h, i (6, 7, 8) | Row 31 | 32 |
| left_edge | nxt_c == 0 | a, d, g (0, 3, 6) | Column 0 | 32 |
| right_edge | nxt_c == 31 | c, f, i (2, 5, 8) | Column 31 | 32 |
| Corners | Overlap of 2 Edge Flags | 5 Perimeter Taps | 4 Frame Corners | 4 (Included above) |
| Center (e) | None | Never Masked | Core Pixel | 0 |

Of 1,024 total frame windows, 124 border windows are masked by edge flags, while 900 interior windows bypass masking untouched. Center tap e is never zeroed.

#### Timing Optimization

Edge flags are computed one step ahead in control_fsm using registers (nxt_r, nxt_c). Registering the flags isolates position comparators from the primary MAC datapath, improving worst-negative slack (WNS) from +0.332 ns to +0.742 ns (Fmax increased from 130.4 MHz to 137.8 MHz).

The remaining critical path originates at these registered flag nets:

bottom_edge_reg⟶tap zero mux⟶multiplier⟶prod_r813

This path spans 7 logic levels (7.297 ns delay), where the high fanout (156 endpoints) contributes 1.517 ns of net delay.

### 6.4 Dynamic Operation: Stalls, Drain, and Back-to-Back Frames

- Stall Handling: When valid_in drops, en deasserts. This freezes LB0, LB1, tap registers a…i, and FSM position counters simultaneously. The masked window at the taps holds its state, while win_valid deasserts to insert a bubble into downstream pipeline registers.

- Drain Phase Flushing: Following the 1,024th ingested input pixel, control_fsm forces en high for 34 continuous cycles to flush remaining pipeline data. Post-frame input data entering tap row g,h,i during row-31 evaluation is masked out by bottom_edge and right_edge.

- Back-to-Back Frame Continuity: The hardware requires no inter-frame reset. When Frame 2 processing begins, residual tail data from Frame 1 residing in LB0/LB1 occupies top-row taps a,b,c. Asserting top_edge during Row 0 of Frame 2 zeroes these taps, preventing cross-frame corruption.

### 6.5 Utilization and Verification Summary

#### Resource Utilization

- LUT Allocation: 132 routed LUTs (116 combinational logic/masking + 16 SRL shift registers).

- Flip-Flops: 117 FFs (including 72 tap bits).

- Block RAM / Distributed RAM: 0 tiles.

#### Unit-Level Verification (tb/tb_window_gen_unit.sv)

Verified via a cycle-accurate software shadow model evaluating four test scenarios:

- Linear Ramp Stimulus: 116 continuous enabled cycles checked every cycle against shadow state.

- Stall Insertion: 5-cycle en deassertion verifying all 9 tap registers freeze without state drift.

- Edge Flag Isolation: Individual assertions of top_edge, bottom_edge, left_edge, and right_edge.

- Corner Masking: Verification of corner flag overlaps, confirming zero-padding on 5 perimeter taps while tap e remains untouched.

#### Integration Verification (tb/tb_top_window.sv)

Evaluated inside the full top-level hardware wrapper against reference vectors (golden_model/vectors/hramp_windows_same.hex):

- Verified all 1,024×9-byte tap states per frame against reference outputs across 22 full regression runs.

- Confirmed exact 34-cycle fill latency, 1,024 window outputs per frame, and zero spatial alignment or index errors.